Lecture No. 2

-Three dimensional stresses and Strains-

2-1 General.

Consider a cube of infinitesimal dimensions shown in figure (1), All stresses acting on this cube are identified on the diagram. The subscripts (τ) are the shear stress, associate the stress with a plane perpendicular to a given axis, the second designate the direction of the stress, i.e.

τxy

Face                  Direction

The stress symbols in figure (1),

shows that three normal stresses: -

σx =  τxx, σy = τyy , σz = τzz

and six shearing stresses, τxy , τxz ,

τyx , τyz , τzx , τzy.                                                       Figure (1)

The force vector (P) has only three components Px , Py and Pz.

PxPyPz

And stress vector: -

σxτxyτxzτyxσyτyzτzxτzyσz   =   τxxτxyτxzτyxτyyτyzτzxτzyτzz                                           ..….2.1

This is a matrix representation of the stress tensor. It is a second –rank tensor requiring two indices to identify its elements or components. A vector is first- rank tensor, and scalar is a zero tensor.

Sometimes, for brevity, a stress tensor is written in identical natation as τij , where i, j and k designations x, y and z.

The stress tensor is symmetric, i.e. τij=  τji  or

τxy=  τyx

τxz=  τzx                                                                                                  	             …2.2

τyz=  τzy

2-2 Two dimensional stress (Biaxial stress): -

For a two dimensional case of plane stress where (σ3=0).

σy

Where:-

τxy

σ= Normal stress

τ = Shear stress

σx

σij= σxτxy0τyxσy0000

2-3 Three dimensional stress (Triaxial stress): -

There are nine components of stress. Moment equilibrium can be used to reduce the number of stress components to six.

y

x

σy

τxy=  τyx

z

τxz=  τzx

τyz

τyx

τxy

τyz=  τzy

τxz

σx

σx

Stresses in 3D is represented by vector

tensor: -

σy

σijk = σxτxyτxzτyxσyτyzτzxτzyσz

A homogenous linear equation has a solution only if the determinant of the coefficient matrix is equal to zero this called Eigenvalue problem. Such as

σxτxyτxzτyxσyτyzτzxτzyσz  =  000                                                    		 	…2.3

In case that the equation is in eigenvalue problem such as: -

σxτxyτxzτyxσyτyzτzxτzyσz  = 0                                                      			...2.4

The determinant can be expanded to yield the equation

σ3-I1σ2+I2σ-I3=0                              	  .		..2.5

Where I1, I2 and I3 are the first, second and third invariants of the Cauchy stress tensor.

I1=σx+σy+σz                                                                            	 ..2.6

I2=σxσy+σxσz+σyσz-τxy2-τxz2-τyz2                              	...2.7

I3=σxσyσz+2τxyτxzτyz-σxτyz2-σyτxz2-σzτxy2    	...2.8

There are three roots the characteristic equation 2.5, σ1, σ2 and σ3. Each root is one of the principal stresses. The direction cosines can be found by substituting the principal stresses into the homogenous equation 2.3 and solving. The direction cosines define the principal direction or planes.

σin

σ max ≥ σin ≥ σmin

σmin

σ max

σ1≥ σ2 ≥ σ3

τ12=σ1-σ22                  ……2.9

τ23=σ2-σ32                   ..….2.10

τ13=σ1-σ32  =τmax        ..….2.11

2-4 Poisson’s Ratio (υ): -

Biaxial and Triaxial deformation another type of elastic deformation is the change in transverse dimensions accompanying axial tension or compression.

Experiments show that if a bar is lengthened by axial tension, there is a reduction in the transverse dimensions. Simeon D. Poisson showed in 1811 that the ratio of the unit dimensions or strain in these directions is constant for stresses within the proportional limit,

Poisson’s Ratio (υ) =-Lateral strainLongitudinal Strain                             	      …2.12

=-εyεx=-εzεx

Where: -

εx=Strain in x-direction.

εy=Strain in y-direction.

εz=Strain in z-direction.

δd

d

Note: - Minus sign indicates a decrease in transverse dimension position as in the case of tensile elongation.

δL

εL=δLL

L

εd=δdd

∴ν=-δd/dδL/L

2-5 Three Dimensional Strains: -

For Tri-axial stress state,

If σy,σz=0→εx=σxE

If σx,σz=0→εy=σyE  and εx=-υσyE

Now, If  σx,σy=0→εz=σzE  and εx=-υσzE

∴εx=σxE-υσyE-υσzE

∴εy=σyE-υσxE-υσzE                              			      …2.13

∴εz=σzE-υσxE-υσyE

So, the principal strains in terms of principal stresses,

∴ε1=1E(σ1-νσ2-υσ3)

∴ε2=1E(σ2-νσ1-υσ3)                              	               …2.14

∴ε3=1E(σ3-νσ1-υσ2)

…………………………….End…………………………