Classification of computer architecture

Flynn’s taxonomy:

Classification of computer systems according to Michael Flynn (1966) is based on the presence of single or multiple streams of instruction and data:

Instruction stream: a sequence  of instructions executed by a processor.

Data stream: a sequence of data required by an instruction stream.

SISD:  Execution of instruction in SISD processors is illustrated bellow:

The SISD organization

• Instructions are executed sequentially.

• This is the oldest and until recently, the most common form of computer

• Examples: most PCs, single CPU workstations and mainframes

SIMD:  Execution of instruction in SIMD processors is illustrated bellow:

The SIMD organization

• A type of parallel computer

• Best suited for specialized problems  such as image processing and vector computation.

MISD: Execution of MISD processor is illustrated bellow:

The MISD organization

• This architecture is also known as systolic arrays for pipelined execution of specific instructions

• Few actual examples of this class of parallel computer have ever existed. One is the experimental Carnegie-Mellon C.mmp computer (1971).

MIMD  Execution of MIMD processor is illustrated bellow:

The MIMD organization

• As shown in  the above figure  there are different processor each processing different task.

• Examples: most current supercomputers, networked parallel computer .