Chapter Two

Z-Transform

# A- Discrete Time Signals:

The discrete time signal x(n) is obtained by taking samples of the analog signal xa (t) every Ts seconds as shown in Figure below.

Analog  signal

xa(t)

Discrete time  signal

x(n)=xa(nTs)

fs=1/Ts

Sampler

0

xa(t)

t

Ts  2Ts  3Ts …..5Ts

5

1

2

3

4

0

x (n)

n

xn=xanTs                   -∞<n<∞

t=nTs=n1fs =nfs

where fs is sampling frequency and  Ts is sampling period.

Example 1:  Consider an analog sinusoidal signal  xat=Acos2πft+θ  which, when sampled periodically at a rate fs = 1/Ts samples per second, yields

xn=xanTs=Acos2πfnTs+θ=Acos2πnffs+θ,

where ω=2πf is analog frequency  and

ωs=ωTs=2πffs  is digital frequency

# Sampler Mathematical Model:

0       Ts     2Ts  3Ts

……

δTst

t

The sampler can be defined as an impulse separator with intervals of Ts and amplitude of xanTs.

xat

xn

δTst

δTst=n=0∞δ(t-nTs)

xn=xatδTst=xa0δt+xaTsδt-Ts+xa2Tsδt-2Ts+…

xn=n=0∞xanTsδ(t-nTs)

# Sampling Theorem

To recover the analog signal from its sampled values the sampling frequency must be greater or equal Nyquist rate. fs≥2fm,where fm is the maximum frequency of sampled signal, and fs is the minimum rate to cover the original signal  fs=2fmis at the Nyquist rate.

# Example 2:  Consider the analog signal:

xat=3cos50πt+10sin300πt-cos100πt, what is the Nyquist rate for this signal?

# Solution:

f1=25Hz, f2=150Hz, f3=50H,

Therefore the maximum frequency fm=150Hz, and fs≥2fm→fs≥300Hz, then, the Nyquist rate is 300Hz.

∴xn=xanTs=3cos0.167πn+10sinπn-cos0.33πn

δn

n

Some Elementary Discrete Time Systems:-

- Unit Sample Sequence

δn-no

n

no

1

δn=1         n=00         n≠0

δn-no=1         n=no0         n≠no

1

0     1    2    3

un

n

. . . .

Unit Step Sequence

un=1         n≥00         n<0, un=0, 0, 0, 1, 1, 1, …↑

un-no=1         n≥no0         n<no

- Real valued exponential sequence

xn=anun, a∈R       (Positive time sequence),

for example, xn=0.9nun      0≤n≤10 .

xn=-bnu-n-1, b∈R      (Negative time sequence).

- Complex valued exponential sequence

xn=e(σ+jωo)n, for example,n=e(2+j3)n=e2nej3n .

- Sinusoidal Sequence

xn=sinnωo+θun or xn=cosnωo+θun.

- Unit Ramp Sequence

xn=nun, .

- Unit Sample Synthesis

xn=k=-∞∞xkδn-k;  k=-∞∞xkδk=x(0)

- The geometric series

n=0∞an=1+a+a2+…=11-a  ,  a<1

n=0N-1an=1-aN1-a    , a<1

n=0∞nan=a(1-a)2  , a<1

T[x(n)]

h(n)

x(n)

y(n)

Response of LTI Systems to Arbitrary Inputs:

# )The Convolution Sum(

# The input is the arbitrary signal x(n) that is expressed as a

sum of weighted impulses:

xn=k=-∞∞xkδn-k

The response of the system to x (n) is the corresponding sum of weighted outputs

y(n)=T[x(n)]=Tk=-∞∞xkδn-k

= k=-∞∞xkT[δn-k]]

Since the impulse response h(n)=T[δ(n)] then by the time-invariance property, the response of the system to the delayed unit sample sequence δ(n-k) is

h(n-k)=T[δ(n-k)]

Then the output response of the LTI system as a function of the input sign al x(n)  and the impulse response h(n) can be expressed as

yn=xn⊛hn=k=-∞∞xkhn-k=k=-∞∞hkxn-k

The equation above is called convolution sum.

To summarize, the process of computing the convolution between x(k) and h(k) involves the following four steps:

1. Folding. Fold h(k) about k = 0 to obtain h(-k).

2. Shifting. Shift h(-k) by no to the right (left) if no is positive (negative), to obtain h(n0 - k).

3. Multiplication. Multiply x(k) by h(n0 -k) to obtain the product sequence

vno(k) =x(k)h(n0 -k).

4. Summation. Sum all the values of the product sequence vno(k) to obtain the

value of the output at time n = n0.

Example 3:

hn

n

0

3

-1

1

2

3

4

2

1

Find x(n)⊛h(n)by using a graphical method?

xn

n

0

0.5

-1

-2

1

2

3

4

# Solution:

yn= x(n)⊛h(n)=k=-∞∞xkhn-k

hn-k

k

n

3

n-1

n-2

n-3

2

1

h-k

k

0

3

-1

-2

-3

2

1

- n≤-1        yn=0

y0

0

1.5

h-k

k

0

3

-1

-2

-3

2

1

n=0

xk

k

0

0.5

1

2

y0= k=-∞∞xkh-k=1.5

- n=1

y1

0

1.5

1

1

h1-k

k

1

3

0

-1

-2

2

1

xk

k

0

0.5

1

2

y1= k=-∞∞xkh1-k=2.5

y2

0

1.5

1

1

0.5

2

n=2

xk

k

0

0.5

1

2

h2-k

k

2

3

1

0

-1

2

1

y2= k=-∞∞xkh2-k=3

y3

0

1

1

0.5

2

n=3

h3-k

k

3

3

2

1

0

2

1

xk

k

0

0.5

1

2

y3= k=-∞∞xkh3-k=1.5

y4

0

1

0.5

2

h4-k

k

3

3

2

1

0

2

1

4

n=4

xk

k

0

0.5

1

2

y4= k=-∞∞xkh4-k=0.5

- n≥5        yn=0

∴yn=0, 0, 1.5, 2. 5, 3, 1.5, 0.5, 0, 0, …

yn

n

3

3

2

1

0

1.5

2.5

4

-1

-2

5

6

1.5

0.5

h(n)={ 1,2,1,-1}

HW : the impulse response of a linear time invariant system is

# Determine the response of the system to the input signal

x(n)={ 1,2,3,1}

1

uk

k

. . . .

Example 4: Suppose a linear shift invariant system with input x(n) and output y(n) is characterized by its unit sample response h(n)=an u(n) for 0 < a < 1. Find the response y(n) of such system to the input signal x(n)=u(n).

# Solution:

1

n

un-k

k

. . . .

yn=k=-∞∞hkx(n-k)

=k=-∞∞aku(k)u(n-k)

1

n

uku(n-k)

k

. . . .

= k=0nak=1-an+11-a u(n)

# System Properties

1-Linear and nonlinear: A system is said to be linear if

T[a1x1(n)+a2x2(n)]=a1T[x1(n)]+a2T[x2(n)]

For any two input x1(n) and x2(n) and for any complex constant a1 and a2.

# 2- Shift invariance (Time invariance) : T[x(n-n0)]=y(n-n0)

# 3- Causality: If has zero values for n<0.

4-Stability: the system is stable if for any bounded input |x(n)|≤B<∞ ,the output will be bounded  |y(n)|<∞  (BIBO)

# For LTI system, stability is S=n=-∞∞hn<∞

Example 5: a given system is represented by the following input output formula

y(n)=T[x(n)]=n x(n). Show whether the system is linear or nonlinear and time varying or not.

# Solution:

Linearity: let x(n)= a1x1(n)+a2x2(n)

∴y(n)=n[a1x1(n)+a2x2(n)]= a1 n x1(n)+a2 n x2(n)= a1T[x1(n)]+a2T[x2(n)]

∴ system is linear

Shift invariance: let x(n)=u(n)→ y(n)=nu(n)

If x(n)=u(n-2) → y(n)=n u(n-2) ≠ y(n-2)

∴ system is shift varying

Example 6 :  a LTI system is characterized by h(n)=an u(n) . Classify this system if causal or non-causal an stable or unstable.

# Solution:

# Causality:  the system is causal since h(n) is zero for n<0

# Stability:  S=n=-∞∞anu(n)=n=0∞an=11-|a|<0

If |a|<1 the system is BIBO and hence it is stable since S is finite.

# If |a|≥1 is unstable since S diverges

B- The Direct Z-Transform

# From Fourier Transform to Z- Transform:

The time representation of the discrete signal is:

xn=n=0∞xanTsδ(t-nTs)

By taking the Fourier transform for both sides

Fxn=n=0∞xanTs Fδ(t-nTs)n=0∞xanTse-jωnTs

# ∴Fx(n)=n=0∞x(n)e-jnωs

Where ωs=nω

# The Z-transform is defined by replacing with z to ejωs yield

Xz=Zxn=n=0∞x(n)e-jnωsejωs=z=n=-∞∞x(n) z-n

Zx(n)=n=-∞∞x(n) z-n

The z in the above definition is a complex variable and the set of z values for which the summation converges is called the region of convergence (ROC) for the transform. In general this region is bounded by

Rx-<z<Rx+,

where Rx- is lower (minimum) limit of this region may be zero, and Rx+ is upper (maximum) limit of it may be infinity.

Example 7:  Find the Z-transform including the region of convergence of

xn=an         n≥00           n<0

# Solution:

# Xz=n=-∞∞xnz-n=n=-∞∞anu(n)z-n=n=0∞(az-1)n

# By using the geometric series

n=0∞xn=1+x+x2+…=11-x, x<1

# ∴Xz=n=0∞(az-1)n=11-az-1=zz-a

The above result converges if az-1<1 or z>a

Most useful z-transforms can be expressed in the form,Xz=P(z)Q(z),where P(z) and Q(z) are polynomials in z represent the numerator and denominator of X(z) respectively. The values of z for which P(z) and X(z) equal to zero are called the zeros of X(z), and the values with Q(z) = 0 and X(z) goes to infinity are called the poles.

Zero at origin

Real

Imaginary

Pole at Z=a

Z>a

Boundary of region of convergence (ROC)

Example 8: Find the Z-transform including the region of convergence of

xn=-bnu(-n-1)

# Solution:

Xz=n=-∞∞xnz-n=n=-∞∞-bnu(-n-1)z-n=-n=-∞-1(bz-1)n

By letting n=-m in the above summation, that is changed the variables, and since interchanging the order of summation has not changed the sign, X(z) becomes

# Xz=-m=1∞zbm=1-m=0∞(zb)m

The last step was obtained by adding the m=0 term to the sum and subtracting it from the total. By using the geometric progression formula to evaluate the infinite sum, X(z) becomes.

# Xz=1-11-zb=zz-b

and hence the ROC of this transform is z/b<1→Z<b

Zero at origion

Real

Imagenary

Pole  at Z=b

Z<b

Boundary of region of convergence (ROC) Z=b

Example 9:  Find the Z-transform including the region of convergence of

xn=anu(n)-bnu(-n-1)

# Solution:

# Xz=n=-∞∞xnz-n=n=-∞∞anu(n)-bnu(-n-1)z-n

Xz=n=-∞∞anu(n)z-n-n=-∞∞bnu-n-1z-n=zz-a+zz-b

From previous two examples the regions of convergence of them arez>a and z<b respectively. And the final region of convergence of the result in this example is the intersection between these two regions.

ROC is z>a∩z<b

For drawing this region, it depends upon the values of the two variables a and b.

- If b<a, the above intersection is the empty set, i.e. the transform doesn't converge.

- If b>a, the transform converges in the annular region as shown in  Figure below.

Real

Imagenary

b

a

ROC

Real

Imagenary

a

b

Example 10: Find the Z-transform including the region of convergence of

1) xn=δn                        2) xn=δn-k k>0

Solution:

1) Xz=n=-∞∞xnz-n=n=-∞∞δnz-n=z0=1,  ROC is ALL values of z

2) XZ=n=-∞∞xnz-n=n=-∞∞δn-kz-n=m=-∞∞δmz-m-k

=z-km=-∞∞δmz-m=z-k,    ROC z>0

Example 11:  Find the Z-transform including the region of convergence of

xn=un

Solution:

Xz=n=-∞∞xnz-n=n=-∞∞unz-n=n=0∞z-n=11-z-1

Zun=zz-1  ,ROC z>1

Properties of Z-transform

- Linearity

If  X1z=Zx1n    with ROC R1-<z<R1+

X2z=Zx2(n)   with ROC R2-<z<R2+

Then

Za1x1n+a2x2n=a1X1z+a2X2z

ROC= ROC1∩ROC2Intersection

- Translation {Shifting}

Zx(n)=X(z) with ROC Rx-<z<Rx+

Then

Zx(n-no)=z-noXzwith the same ROC

Proof:

Zx(n-no)=n=-∞∞xn-noz-n

Let n-no=m

Zx(m)=m=-∞∞xmz-m-no=z-noX(z)

Example 12: Find the Z-transform including the region of convergence of

xn=un-4

Solution: Xz=Zun-4=z-4Zun=z-4zz-1=z-3z-1       , ROC  z>1

- Multiplication by an exponential

Zanx(n)=X(z)Z→Za=Xza       with ROC aRx-<z<aRx+

Ze±anxn=Xzz→ze∓a=Xze∓a

with ROC e±aRx-<z<e±aRx+

Proof:

1) Zanx(n)=n=-∞∞anxnz-n=n=-∞∞xn(a-1z)-n=X(za)

2) Zeanxn=n=-∞∞xneanz-n=n=-∞∞xn(e-az)-n=X(zea)

Example 13: Find the Z-transform including the region of convergence of

xn=neanun

Solution:

Znun=n=-∞∞nunz-n=n=0∞nz-n=z-11-z-12=z(z-1)2

ROC Z-1<1→Z>1

∵Zeanxn=X(zea)

∴Zneanun=ze-a(ze-a-1)2      , ROC z>ea

- Multiplication by n (Ramp)

Znxn=-zdXzdz       , with ROC Rx-<z<Rx+ (ROC no change)

Example 14: Find the Z-transform including the region of convergence of

Zneanun

Solution: ∵Zeanun=zz-ea       and       Znxn=-zdXzdz

Zneanun=-zdzz-eadz=-zz-ea.1-z.1z-ea2=zeaz-ea2

- Initial value theorem

x0=limn→0xn=limz→∞Xz

- Final value theorem

x∞=limn→∞xn=limz→1(1-z-1)Xz

- Discrete convolution

x1(n)⊛x2(n)=k=-∞∞x1kx2n-k

Zx1(n)⊛x2(n)=X1zX2z      ,with ROC, Z∈R1∩R2

Proof:

Zx1(n)⊛x2(n)=n=-∞∞k=-∞∞x1kx2n-kZ-n

=k=-∞∞x1kn=-∞∞x2n-kz-n=X2(z)k=-∞∞x1kz-k=

= X1zX2z

HW: Find Z{u(n)⊛nu(n)}

- Time reversal

xn↔Xz     , ROC r1<z<r2

x-n↔Xz-1      ,ROC 1r2<z<1r1

HW: Find Z{-n u(-n)}

Example 15: Find the Z-transform including the region of convergence of

xn=cos(ωon)u(n)

Solution:

Zejωon+e-jωon2=12Zejωon+e-jωon=12zz-ejωo+zz-e-jωo

=12z2-ze-jωo+z2-zejωo(z-ejωo)(z-e-jωo)=122z2-z(ejωo+e-jωo)z2-ze-jωo-ze-jωo+1

=122z2-2zcos(ωo)z2-z(e-jωo+e-jωo)+1=z2-zcosωoz2-2zcosωo+1,

ROC z>e±jωo→z>1

Example 16: Find the Z-transform including the region of convergence of

xn=(n-2)a(n-2)cos(ωo(n-2))u(n-2)

Use cos(ωon)u(n)=z2-zcosωoz2-2zcosωo+1 and any necessary properties.

Solution:

Zx(n)=z-2 Zna(n)cos(ωon)u(n)  by using shift property

=z-2-zddzZa(n)cos(ωon)u(n) by using multiplication by n property

=-z-1ddzZcos(ωon)u(n)z→za by using multiplication by an property

ROC Z>a,
Zcos(ωon)u(n)=z2-zcosωoz2-2zcosωo+1

Zcos(ωon)u(n)z→za=a-2z2-a-1zcosωoa-2z2-2a-1zcosωo+1=z2-azcosωoz2-2azcosωo+a2

Zx(n)=-z-1ddzz2-azcosωoz2-2azcosωo+a2=…

H.W:

- Z2cos0.2π(n-1)u(n)

- Ze-3nsin(πn6)u(n)

- Zsinh(ωon)u(n)

C- The Inverse of Z-transform Z-1x(n):-

Example 17: Determine the signal xn for Xz=ln(1+az-1), z>a

Solution:

using Znxn=-zddzX(z)   ,  ddxln(u)=1ududx

ddzXz=11+az-1-az-2→-zddzXz=az-11+az-1

ROC -az-1<1→ z>a

Take the inverse of Z-transform for both sides to get:

nxn=Z-1az-11+az-1→nxn=a-an-1u(n-1)

∴xn=1n-1n-1anun-1

There are many methods to find the inverse of Z-transform

1- Partial fraction method

i) Distinct poles

Xzz=A1z-p1+A2z-p2+…+ANz-pN

Ai=limz→piz-piX zz

Example 18: Find xn for    Xz=11-1.5z-1+0.5z-2 for the following ROC's:

a) z>1   ,      b)  z<0.5     and       c) 0.5<z<1

Solution:

Xz=z2z2-1.5z+0.5→Xzz=z(z-1)(Z-0.5)=A1z-1+A2z-0.5

A1=limz→1z(z-0.5)=2,    A2=limZ→0.5z(z-1)=-1

1

Real

Im

ROC

Xzz=2z-1-1z-0.5,  Xz=2zz-1-zz-0.5

- ROC z>1

xn=2un-0.5nu(n)

0.5

1

Real

Im

ROC

- ROC Z<0.5

0.5

1

Real

Im

ROC

xn=-2u-n-1+0.5nu(-n-1)

- ROC 0.5<z<1

xn=-2u-n-1-0.5nu(n)

ii) Multiple order poles

IfXzz=do+d1z+d2z2+…+dM-1zM-1bo+b1z+b2z2+…+bMzM

=A1k(z-zr)k+A1k-1(z-zr)k-1+…+A11z-zr+j=k+1MAjz-zj

Then

A1j=1k-j!dk-jdzk-j(z-zr)kX(z)zz=zr, j=1, 2, 3, …, k

Aj=(z-zj)X(z)zz=zj, j=k+1, k+2, …, M

Example 19: Find xn) for Xz=1(1+z-1)(1-z-1)2

Solution:  Xz=1(1+z-1)(1-z-1)2=z3(z+1)(z-1)2

X(z)z=z2(z+1)(z-1)2=A12(z-1)2+A11z-1+A3z+1

A12=limZ→1z2(z+1)=12, A11=limZ→12zz+1-z2(z+1)2=34

A3=limZ→-1z2(z-1)2=14

X(z)=12z(z-1)2+34zz-1+14zz+1

x(n)=12nu(n)+34u(n)+14-1nu(n)

Notes:

- anu(n)is a right hand sequence (causal or positive time sequence).

- -anu(-n-1)is a left hand sequence (anti-causal or negative time sequence).

Example 20: Find the inverse Z-transform of

Xz=z2+z(z-0.5)3(z-0.25) for the ROC z>12

Solution:

Xzz=z+1(z-0.5)3(z-0.25)

=A13(z-0.5)3+A12(z-0.5)2+A11(z-0.5)+A4(z-0.25)

A13=limz→0.5z+1(z-0.25)=6, A12=limz→0.5z-0.25-z-1(z-0.25)2=-20

A11=12!limz→0.5d2dz2z+1z-0.25=12!limz→0.5ddz-1.25(z-0.25)2=12limz→0.52.5z-0.25(z-0.25)4=80

A4=limZ→0.25z+1(z-0.5)3=-80

Xz=6z(z-0.5)3-20zz-0.52+80zz-0.5-80z(z-0.25)

xn=6nn-1212n-2un-20n12n-1un+8012nun-8014nu(n)

Notes:   At at ROC Z>a

- Z-1zz-a=anu(n).

- Z-1z(z-a)2=nan-1u(n)

- Z-1z(z-a)3=n(n-1)2!an-2u(n)

- In general Z-1z(Z-a)k=nn-1n-2…(n-k+2)(k-1)!an-k+1u(n)

HW:  Using the partial fraction method to find x(n) for

  - Xz=z(z2-4z+5)(z-3)(z-1)(Z-2) for ROC's (a) 2<z<3, (b) z>3, (c) z<1

  - Xz=z3+2z2z-0.52(z+34)for ROC  z>34

  - Xz=z3-2z2z-0.5(z+14)for ROC  14<z>12

2- Power series inverse method (division)

Example 21:  Find the inverse Z-transform by division method for

Xz=z3z2-4z+1    for the ROC's az>1, bz<13

Solution:

- For ROC z>1, we must divide to obtain negative power of z                    since  z>1 indicates a right hand sequence.

3z2-4z+10.5

Z

13z-1+49z-2

∓z±43∓13z-1

43-13z-1

∓43±169z-1∓49z-2

139z-1-49z-2etc

Xz=13z-1+49z-2+…as compare with

Xz=…+x-2z2+x-1z1+x(0)z0+x(1)z-1+x2z-2+…

We can recognize that x1=13 , x2=49

xn=13δn-1+49δn-2+…

- For ROC z<13, must divide to get positive power of z since z<13 indicates a left hand sequence, for negative n.

1-4z+3z2

z

z+4z

∓z±4z2∓3z3

4z2-3z3

∓4z2±16z3∓12z4

13z3-12z4 etc

Xz=z+4z2⇛ x-1=1 , x-2=4

xn=δn+1+4δn+2+…

3- Discrete convolution

Example 23: Find the inverse Z-transform by using discrete convolution method  for  Xz=0.632z(z-1)(z-0.368)

Solution:

Xz=0.632z(z-1)(z-0.368)=0.632z(z-1)∙z-1.zz-0.368=X1z.X2z

x1n=Z-1X1z=Z-10.632z(z-1)⇛x1n=0.632u(n)

x2n=Z-1X2z=Z-1z-1.zz-0.368⇛x2n=0.632n-1u(n-1)

∴xn=x1n⊛x2n=0.632u(n)⊛ 0.632n-1u(n-1)

# HW: Determine the response y(n), n≥0 of the system described by the second order difference equation:        y(n)-4y(n-1)+4y(n-2)=x(n)-x(n-1)

# when the input is x(n)=(-1)n u(n) and the initial conditions are                           y(-1)=y(-2)=0.

# Tutorial Sheet No. 2

# Z- Transform

# 1. Determine Z-transform including region of convergence

# a) x(n)=n an u(-n-1)

x(n) = {1, 0, -1,0,1, -l)

# b)

# c) xn=(13)n        for n≥0(12)-n    for n<0

# 2. Compute the convolution of the following signals

# x1n=(13)n        for n≥0(12)-n    for n<0      and          x2n=12nu(n)

# 3. Using long division, determine the inverse z-transform of

# Xz=1+2z-11-2z-1+z-2

# 4. Determine the causal signal x(n) if its z-transform X(z) is given by:

# a) Xz=1+3z-11+3z-1+2z-2

# b)  Xz=11-z-1+0.5z-2

# c) Xz=2-1.5z-11-1.5z-1+0.5z-2

# 5. Given Xz=z(z+1)z2-3z+2  find x(n) for the following ROC using partial fraction method:     a) ROC          |z|>2

# b) ROC          |z|<1

# c) ROC     1<|z|<2

# 6. Determine the response of the system with impulse response h(n)=an u(n) to the input signal x(n)=u(n)-u(n-10).

0.5

-0.5

0.5

-0.5

-1

Im(z)

Re(z)

7. Let x(n) be a causal sequence with z-transform X(z) whose pole-zero plot is shown below. Sketch the pole-zero plots and the ROC of the following sequence:

# a) x1(n)=x(2-n)

# b)x2(n)=ejπ3n x(n)

8. We want to design a causal discrete-time LTI system with the property that if the input is xn=12nun-1412n-1un-1

# Then the output is yn=13nun

# Determine the impulse response h(n) and the system function H(z).

# 9. Consider the causal system

# yn=-a1yn-1+b0xn+b1x(n-1)

# Determine:

# (a) The impulse response

# (b) The step response

# (d) The response to the input      xn=cos(w0n)     0≤n<∞